PWM Controller - Technical Specifications

1. Project Overview

Project: 8-bit PWM Controller
Description: Digital Pulse Width Modulation controller with glitch-free operation
Technology: Verilog HDL
Simulation: Vivado IDE
Test Method: Use "Run All" command in Vivado for simulation

2. Module Interface

2.1 Port Definitions

verilog

module pwm_controller (

input wire clk,           // System clock (50 MHz)

input wire reset,         // Active-high synchronous reset

input wire [7:0] duty,    // Duty cycle value (0-255)

input wire enable,        // Module enable control

output reg pwm_out        // PWM output signal

);

2.2 Signal Description

| Signal | Width | Direction | Description |
| --- | --- | --- | --- |
| clk | 1-bit | Input | System clock (50 MHz) |
| reset | 1-bit | Input | Active-high synchronous reset |
| duty | 8-bit | Input | Duty cycle value (0-255) |
| enable | 1-bit | Input | Module enable (1=enabled, 0=disabled) |
| pwm_out | 1-bit | Output | PWM output signal |

3. Technical Parameters

3.1 Resolution & Range

| Parameter | Value | Description |
| --- | --- | --- |
| Resolution | 8-bit | 256 discrete steps |
| Counter Range | 0-255 | Free-running counter period |
| Duty Cycle Range | 0-255 | 0% to 99.6% |
| Step Precision | ~0.4% | 1/256 of full scale |

3.2 Timing Specifications (50 MHz Clock)

| Parameter | Value | Calculation |
| --- | --- | --- |
| Clock Frequency | 50 MHz | External input |
| Clock Period | 20 ns | 1/50 MHz |
| Counter Period | 256 cycles | 2^8 steps |
| PWM Frequency | 195.3125 kHz | 50 MHz / 256 |
| PWM Period | 5.12 μs | 256 × 20 ns |

3.3 Duty Cycle Timing Examples

| Duty Value | Percentage | High Time | Low Time |
| --- | --- | --- | --- |
| 0 | 0% | 0 ns | 5120 ns |
| 64 | 25% | 1280 ns | 3840 ns |
| 128 | 50% | 2560 ns | 2560 ns |
| 192 | 75% | 3840 ns | 1280 ns |
| 255 | 99.6% | 5100 ns | 20 ns |

4. Functional Description

4.1 Operating Principle

text

PWM Output Logic:

pwm_out = 1 when counter < duty_reg

pwm_out = 0 when counter >= duty_reg

4.2 Block Diagram

text

+-------------------------------+

|        PWM Controller         |

|                               |

--> | clk    [8-bit Counter]        | --> pwm_out

--> | reset  [0->255->0]            |

--> | duty   [Duty Register]        |

--> | enable [Comparator]           |

|        counter < duty_reg?    |

+-------------------------------+

4.3 Key Features

- Synchronous Design: All operations clock-synchronized

- Glitch-Free Updates: Duty cycle changes at period boundaries

- Enable Control: Output forced LOW when disabled

- Clean Reset: Counter and output reset to known state

5. Simulation & Verification

5.1 Testbench Structure

text

Testbench Files:

- tb_pwm_controller.v (Basic functionality)

- tb_pwm_controller_advanced.v (Advanced measurements)

5.2 Test Scenarios

| Test Case | Description | Expected Result |
| --- | --- | --- |
| Reset Test | Apply reset signal | Counter=0, PWM=0 |
| 0% Duty | duty=0 | PWM always LOW |
| 50% Duty | duty=128 | PWM HIGH for 128 cycles |
| 100% Duty | duty=255 | PWM always HIGH |
| Enable/Disable | Toggle enable | PWM follows enable state |
| Dynamic Changes | Change duty during operation | Smooth transitions |

5.3 Simulation Instructions

- Add Files: Add all Verilog files to Vivado project

- Set Top Module: Set testbench as top module for simulation

- Run Simulation: Use "Run All" command in Vivado

- Verify Results: Check console output and waveforms

Important: Always use the "Run All" command in Vivado for proper simulation execution.

6. Performance Characteristics

6.1 Resource Utilization (Estimated)

| Resource | Count | Description |
| --- | --- | --- |
| Flip-Flops | 16 | 8-bit counter + 8-bit duty register |
| LUTs | 8 | Comparator and control logic |
| Maximum Frequency | >100 MHz | Limited by target FPGA |

6.2 Timing Constraints

- Setup Time: 2 ns

- Hold Time: 1 ns

- Clock-to-Output: 3 ns

7. Applications

7.1 Typical Use Cases

- LED Dimming: Control brightness with duty cycle

- Motor Control: DC motor speed regulation

- Power Management: Voltage regulation circuits

- Audio Generation: Simple tone generation

7.2 Integration Example

verilog

// Example: LED brightness control

pwm_controller led_pwm (

.clk(sys_clk),

.reset(sys_rst),

.duty(led_brightness),  // 0-255 from user input

.enable(1'b1),

.pwm_out(led_pin)

);

8. Verification Checklist

- Reset functionality verified

- All duty cycle values tested (0, 128, 255)

- Enable/disable operation confirmed

- Glitch-free transitions verified

- Timing requirements met

- Simulation completed using "Run All" command

9. Revision History

| Version | Date | Changes |
| --- | --- | --- |
| 1.0 | 2024-01-01 | Initial specification |
| 1.1 | 2024-01-01 | Added simulation instructions |

Note: For simulation, always use the "Run All" command in Vivado to ensure proper compilation and execution of testbenches.